TECHNICAL REPORT # 4

THE AFAD PROJECT OF MAGNITUDE CALIBRATION

Mehmet Ozyazicioglu, Jens Havskov, Lars Ottemöller, Tugbay Kilic, Kenan Yanik

September 2013

Introduction

This report covers activities during visit of Mehmet Ozyazicioglu, Tugbay Kilic, Kenan Yanik

to UiB in 26-30 August 2013.

The purpose of this visit was to finalize the calibration for Ml magnitude, testing the scale as well as Q determination by QLg method using the data set prepared. We will also compare derived Ml scale magnitudes to Mw calculated by spectral method, and two Mw magnitudes (by moment tensor inversion (Dreger) and spectrum method using average Q, alpha and kappa estimated for whole country).

Status at arrival

The original 112 large events (3.5 < M) had been reprocessed with

- p-phase and WA amplitude readings on all triggered channels and some s-phase readings,

- rms < 2 for all except 2 events.

- All corrupt wave form files had been replaced.

- Some new events had been added to the data set, making the total number of events 164.

- All large events were given magnitude information from external sources (ISK, PDE, MSCE, HRV, GFZ).

- All events were provided with a report file containing

  - Tavel time

  - Wadati diagram

  - Distance ordered waveform plots.

- A new small earthquakes (M < 3.0) data set, well distributed over the country has been added (83 events)

Further checking the data set:

- All events were checked with the Wadati and travel time plots

- Locations of 30 suspicious events were compared to MSCE or PDE locations and phase picks were corrected.

- 4 events which either duplicates or fall too far from the network coverage area and do not locate well were deleted, leaving 160 events in the data set.

- 3 stations missing in the station list were added to STATION0.HYP file.

Epicenters of the old, new and small test data set is seen in Figure 1.

Figure 1. Top left: The earlier test data set of 112 events. Top right: The new test data set of 160 events. Bottom: The new test data set of small events (83 events).

Inversion for a new Ml scale

The 160 reprocessed events were used for the Ml inversion. The ray-path distribution is quite good, see Figure 2.

Figure 2. Left: Ray-Path coverage of the 112 events in the old data set. The circles give the event locations and the triangles indicate station locations. Right: The same for the new test data set of 160 events

Figure 3.  Distribution of data with magnitude and distance.

The Ml scale from the previous inversion of 112 evens was

Ml = log A  +  1.15 * log R  +  0.00141 * R  –  2.12  + S

and the new inversion with 160 events

Whole Country:

Ml = log A  +  1.12 * log R  +  0.00162 * R  –  2.09  + S

When the inversion is done for west and east Turkey separately, the scales come out as:

West:

Ml = log A  +  0.83 * log R  +  0.00237 * R  –  1.59  + S

East:

Ml = log A  +  1.11 * log R  +  0.00165 * R  –  2.07  + S

where R is hypocentral distance (km), A is ground displacement amplitude in (nm). It is seen that using the larger data set (160 events) makes almost no difference as compared to the old data set of 112 events. The scales of east and west Turkey are also very similar (see Figure 4).  As with the previous data set, the distance correction term (-log A0 ) is slightly lower than that of Hutton and Boore (1987), see Figure 4. The new scale  is intermediate between Joyner and Bakun (1984) and the intra-plate scales for Norway (Alsaker, 1991) and the northeastern US (Kim, 1998). It seems that compared to California, the attenuation in Turkey must be slightly smaller leading to Ml magnitude to decrease compared to using the California scale.

Figure 4. Comparison of Ml correction term -log A0 with other scales. This figure includes the inversion of east and west Turkey.

Compared to the Hutton and Boore scale (1987) without station corrections, the overall variance reduction is xx %. This improvement is shown in Figure 5. The figure also shows the reduction of distance dependence, although this partly remains with the new scale.

Figure 5. Left: Individual magnitude residuals as function of distance are compared to the event averages for the Hutton and Boore scale (1987) without station corrections and the magnitude scale derived here. Right: Histogram version of the data. The results are shown for all data and eastern Turkey and western Turkey, respectively.

We can conclude preliminarily that the Ml scale for Turkey will be quite similar to the scale for California by Hutton and Boore (1987), but significantly different from Richter (1935) or Bakun and Joyner (1984). The east and west Turkey scale are so similar that there does not seem to be any reason to use 2 different scales (Figure 4).

Table 1. Average Standart Deviation of with Different Ml Scales

| Model | Av. STD |
| --- | --- |
| Bakun and Joyner (1984) | 0.3334 |
| Hutton and Boore (1987) | 0.2786 |
| This Study (whole country) | 0.2758 |

As seen in table 1, average standard deviation of station magnitudes is minimum with the model obtained in this study, indicating better inversion for current data available.

Horizontal vs Vertical Seismogram Amplitudes in Ml

Testing a few events indicate that Ml will increase by about 0.2 if horizontal component are used. This must be studied in more detail and, if confirmed, the Ml scale can be adjusted accordingly.

Comparison of Dreger moment tensor inversion Mw and spectral Mw

Moment tensor inversion  using the Dreger method had been completed at AFAD on 20 events in the test data set. Since the magnitudes determined by the spectral method still has some uncertainty due to attenuation being uncertain, the spectral Mw was calculated using those 20 events and then compared to the Dreger Mw. Since the moment tensor Mw is determined at low frequencies ( <0.05 Hz), it is assumed to represent the ‘correct’ Mw. For the spectral Mw, the following parameters are used:

Vp=6.2 km/s, Vs=3.6 km/s,

density=3.0 g/cm3,

Q=200f0.6

κ=0.4.

Time window = 30 s .

These are the values found most suitable, see technical report #3. Figure 6 shows the comparison.

Figure 6. Comparison of spectral Mw (DDA) and Mw from the Dreger inversion (DRG). The thick line is the 1:1 relationship.

On average the two magnitudes were the same but there is scatter in the data. However the comparison shows that on average the magnitudes obtained with Dreger moment tensor  solutions are correct.

In report #3 it was shown that comparing Mw from CMT tensor solutions, using larger events, also agreed very well with the Mw from the spectra. So in the magnitude range M = 4-7, the spectral method seems to work reliably and the preliminary Q-values seem reasonable.

Comparing Mw to Ml using the latest Ml scale

For the test data set, Mw was recalculated with the parameters used above and the resultant Mw compared to the new Ml, see Figure 7.

Figure 7.  Local magnitude (LDDA) vs. spectral Mw for large earthquakes (160 events). The thick line is the 1 :1 relationship. No station corrections used.

The average Ml and Mw were 3.9 and 4.3, respectively, a difference in magnitude of 0.4. I is seen that the difference decreases with magnitude and at magnitude 5, the difference is only 0.2. If a correction factor of 0.2 is used due to the horizontal vs vertical amplitude difference magnitudes would then be on average the same for magnitudes above 5 and Ml only 0.2 less than Mw at magnitude 3.

Tests werer also made wıth the small event data set. For these events the same spectral parameters were used as for the large events except that the tıme wındow was 20 s. Using events with magnitude 1.5 to 3.0 the averages for Mw and Ml are 2.87 and 2.24, respectively, a difference of 0.6 (see Figure 8). This is larger than the 0.4 we got for the larger magnitudes

And the correct of 0.2 will not bring the two magnitudes together.

Figure 8.  Local magnitude (LDDA) vs. spectral Mw for small earthquakes (83 events). The thick line is the 1:1 relationship. No station corrections used.

Discussion

The new Ml scale made with a larger data set is very similar to the scale found for the smaller data set. The scale seems reasonable compared to other scales; a slightly lower attenuation in Turkey as compared to California does not sound unreasonable. There is still a significant difference between Ml and Mw, particularly for the smaller events in the test data set. This cannot be explained by anchoring the scale at 100 km instead of the ideally more correct 17 km since the shape of the attenuation curve for Turkey virtually identical to most other scales in the distance range 0-100 km. Thus anchoring at 17 km would not change the magnitude values. The old Ml seem to be more in agreement with Mw than the new Ml (see technıcal report 3) . We consider the Mw from spectra correct on average since they compare well with the Mw determined by moment tenor inversion. The reason for the difference from Mw could then be

- Significant difference of amplitudes on vertical and horizontal components

- Non normal sources in the region (source spectra different)

- The old Ml scale is not a standard Ml scale (see technical report #3)

If we use the correction 0.2 for the vertical/horizontal amplitude difference, the difference is at most 0.2 magnitude units in the magnitude range 3-6. This can be considered acceptable.

It is often observed that for larger events Mw=Ml and this is also what is expected theoretically (Deichman, 2006). For events smaller than about magnitude 3 (region dependent), it is observed that Mw ~0.7 Ml and Ml is underestimated relatively to Mw (Deichmann, 2006). In our study the factor ıs 0.8 for small events and 0.9 for the large eventsç This discrepancy could be attributed to various parameters, however it seems that the main cause is the attenuation and scattering between the source and receiver (Bethmann et al, 2011). This can be explained by the higher frequency content of smaller events relative to larger events at shorter distances and therefore higher attenuation than envisaged in the standard Ml relation using larger events. Ideally therefore the Ml scales should have a magnitude dependent term. In practice, this is rarely done. In the current large event  data set we have few events smaller then 3.5. However as expected,  the magnitude difference will increased more for the smaller events.

If AFAD wants to use a standard Ml, calibrated for Turkey, it must be accepted that Ml will be  smaller them Mw, particularly for smaller events. This is due to the limitation in the Ml scale. If other agencies report an Ml  larger than the Ml given by the new scale, it is like that they do not use a correct Ml scale calibrated for Turkey.  We have e.g. found the SeisComp3 system use a non standard scale which seem to overestimate the Ml values by 0.4 at 300 km distance (reference SeisComp3 manual). On the other hand if AFAD wants to report magnitudes with similar values as the old Ml, the spectral Mw scale can be used.  It is strongly advised to discontinue the use of the old Ml scale since it has little scientific justification, the amplitude readings are non standard and consequently the magnitudes cannot be related to any known standard.

Tasks to be completed by the AFAD team (until next meeting)

- Check systematically for the differences between reading on horizontal and vertical component by reading amplitudes on 10 events on both horizontal components.

- Check for amplitude outliers

- Put in missing OLD Ml (LOLD) values in test data set

References

Bethmann, F., N. Deichmann and P. M. Mai (2011). Scaling Relations of Local Magnitude versus Moment Magnitude for Sequences of Similar Earthquakes in Switzerland. Bull. Seism. Soc. Am. 101, 515-534.

Deichman, N. (2006). Local magnitude, a moment revisited. Bull. Seism. Soc. Am. 96, 1267-1277.

Appendix

Station correction terms to be applied

Code        Correction                               Latitude         Longitude

Station #    1 BOZC   0.193  +/- 0.1679   39.842   26.053

Station #    2 KMR    0.346  +/- 0.2251   40.418   27.069

Station #    3 SART   0.156  +/- 0.1467   40.688   27.178

Station #    4 NEV    0.352  +/- 0.2598   39.954   27.263

Station #    5 YNK   -0.119  +/- 0.1881   40.826   27.397

Station #    6 ATI    0.102  +/- 0.1319   40.083   27.563

Station #    7 BALY   0.069  +/- 0.1396   39.740   27.619

Station #    8 SLV   -0.163  +/- 0.1758   41.073   28.140

Station #    9 BAG   -0.241  +/- 0.1766   38.655   26.852

Station #   10 CAM   -0.069  +/- 0.1951   38.748   27.313

Station #   11 ELBA   0.373  +/- 0.1855   41.147   28.431

Station #   12 AKHS   0.344  +/- 0.1331   38.879   27.814

Station #   13 DURS   0.050  +/- 0.1353   39.601   28.474

Station #   14 YAK    0.214  +/- 0.3562   38.515   27.323

Station #   15 BLN    0.072  +/- 0.1571   38.635   27.822

Station #   16 ESK   -0.300  +/- 0.1844   40.607   28.945

Station #   17 KTT    0.032  +/- 0.5797   38.688   28.093

Station #   18 DGB    0.104  +/- 0.1444   38.052   26.882

Station #   19 DUV    0.001  +/- 0.3361   38.220   27.449

Station #   20 IGD    0.178  +/- 0.1813   40.264   29.201

Station #   21 DEMI   0.139  +/- 0.1450   39.043   28.716

Station #   22 GCAM   0.205  +/- 0.1628   37.714   27.242

Station #   23 KRB   -0.175  +/- 0.3360   38.134   28.423

Station #   24 HAR    0.155  +/- 0.2204   38.341   28.799

Station #   25 AYDN   0.021  +/- 0.1309   37.661   27.879

Station #   26 SUL   -0.041  +/- 0.1535   38.054   28.768

Station #   27 KHAL   0.309  +/- 0.1557   38.370   29.492

Station #   28 BBD   -0.020  +/- 0.1469   37.802   28.853

Station #   29 UZP    0.090  +/- 0.3036   38.031   29.175

Station #   30 DNZL   0.102  +/- 0.1987   37.689   29.046

Station #   31 GOLH  -0.047  +/- 0.1434   37.236   29.559

Station #   32 BOLV   0.074  +/- 0.1257   38.718   30.950

Station #   33 KORT   0.149  +/- 0.1342   37.001   30.350

Station #   34 AKAS   0.231  +/- 0.1442   36.233   29.605

Station #   35 KDHN   0.275  +/- 0.1298   38.521   32.114

Station #   36 BBAL   0.093  +/- 0.1192   39.543   33.123

Station #   37 ELDT  -0.035  +/- 0.1280   40.489   33.427

Station #   38 GAZI   0.042  +/- 0.1463   36.235   32.316

Station #   39 ERMK   0.016  +/- 0.1311   36.641   32.911

Station #   40 GULE  -0.060  +/- 0.1647   37.284   34.777

Station #   41 ERBA   0.343  +/- 0.1375   40.681   36.755

Station #   42 PINB   0.081  +/- 0.3359   38.677   36.403

Station #   43 ANDN  -0.049  +/- 0.1379   37.580   36.345

Station #   44 URLA   0.177  +/- 0.1490   38.360   26.596

Station #   45 CVD   -0.148  +/- 0.1515   37.752   28.107

Station #   46 YNC   -0.281  +/- 0.1787   37.814   28.573

Station #   47 TURN   0.062  +/- 0.1471   36.775   28.244

Station #   48 GDZ    0.463  +/- 0.1725   39.088   29.481

Station #   49 CMH   -0.244  +/- 0.1687   40.012   27.970

Station #   50 KNL   -0.038  +/- 0.1623   40.271   27.526

Station #   51 KRC   -0.068  +/- 0.1709   40.265   28.332

Station #   52 KVK    0.101  +/- 0.1844   40.604   26.888

Station #   53 MADM   0.242  +/- 0.2150   40.654   27.665

Station #   54 BOZ   -0.083  +/- 0.2916   40.535   28.782

Station #   55 KKZ   -0.186  +/- 0.1884   41.127   27.345

Station #   56 KLC   -0.332  +/- 0.1682   40.633   29.398

Station #   57 GBZ    0.034  +/- 0.1510   40.787   29.450

Station #   58 ALT   -0.147  +/- 0.1495   41.088   28.740

Station #   59 KIZK  -0.029  +/- 0.1507   36.480   34.144

Station #   60 CDAG   0.112  +/- 0.1194   39.624   34.372

Station #   61 ILGA   0.006  +/- 0.1477   41.052   33.716

Station #   62 CAY   -0.296  +/- 0.2149   37.172   35.318

Station #   63 HAVZ  -0.081  +/- 0.1248   41.074   35.718

Station #   64 AKCD  -0.442  +/- 0.1355   38.296   37.922

Station #   65 DBOC   0.020  +/- 0.2036   41.345   41.667

Station #   66 ARTV   0.334  +/- 0.2158   41.185   41.928

Station #   67 DAGI   0.011  +/- 0.2032   41.078   41.914

Station #   68 DDEM   0.033  +/- 0.1959   40.892   41.754

Station #   69 DIGO   0.331  +/- 0.2177   40.415   43.374

Station #   70 OLTU   0.024  +/- 0.4106   40.545   41.973

Station #   71 HOMI  -0.271  +/- 0.1777   40.045   41.905

Station #   72 DYDN   0.069  +/- 0.1752   39.544   43.689

Station #   73 TUTA   0.009  +/- 0.2077   39.402   42.814

Station #   74 KOPT  -0.051  +/- 0.1770   40.018   40.497

Station #   75 KELT   0.029  +/- 0.1515   40.149   39.256

Station #   76 TATV  -0.004  +/- 0.3367   38.508   42.267

Station #   77 GEVA  -0.103  +/- 0.1859   38.312   43.057

Station #   78 SUSE   0.029  +/- 0.1623   40.209   38.202

Station #   79 KEMA   0.454  +/- 0.1426   39.269   38.493

Station #   80 FRT   -0.035  +/- 0.1919   38.684   39.198

Station #   81 HAKT  -0.094  +/- 0.2072   37.558   43.707

Station #   82 SIRN  -0.430  +/- 0.2207   37.496   42.413

Station #   83 ELZG  -0.011  +/- 0.1522   38.497   38.985

Station #   84 SCER   0.027  +/- 0.1563   39.861   37.129

Station #   85 GER   -0.119  +/- 0.2106   38.032   39.035

Station #   86 MARD  -0.423  +/- 0.1525   37.313   40.778

Station #   87 KAH   -0.417  +/- 0.3036   37.805   38.610

Station #   88 CLH    0.002  +/- 0.2253   38.016   38.207

Station #   89 ATAB  -0.098  +/- 0.1610   37.470   38.295

Station #   90 KHM   -0.033  +/- 0.2208   37.523   36.991

Station #   91 NAR   -0.086  +/- 0.2315   37.392   37.157

Station #   92 KUZU  -0.234  +/- 0.1273   36.773   37.075

Station #   93 YAYL  -0.223  +/- 0.1815   36.034   36.107

Station #   94 DED    0.103  +/- 0.2250   36.961   34.799

Station #   95 BDRM   0.124  +/- 0.1580   37.065   27.444

Station #   96 MANT   0.009  +/- 0.1483   38.491   28.558

Station #   97 HAS   -0.392  +/- 0.5818   36.772   36.465

Station #   98 BORA   0.275  +/- 0.1260   39.880   30.453

Station #   99 KAMA   0.065  +/- 0.1798   37.186   36.668

Station #  100 HCB    0.093  +/- 0.2253   37.344   36.908

Station #  101 BTAS  -0.165  +/- 0.1235   40.571   31.000

Station #  102 BCAM   0.024  +/- 0.1241   40.816   32.066

Station #  103 BES   -0.337  +/- 0.1947   37.700   37.855

Station #  104 KAST   0.112  +/- 0.1664   41.490   33.929

Station #  105 DIYA  -0.223  +/- 0.1524   37.924   40.121

Station #  106 EUZM   0.316  +/- 0.1691   39.708   39.698

Station #  107 EKAR  -0.050  +/- 0.2798   39.256   42.064

Station #  108 MACK  -0.024  +/- 0.1882   40.943   39.769

Station #  109 DBAD  -0.014  +/- 0.2167   41.018   41.695

Station #  110 TVAN   0.104  +/- 0.2323   38.524   43.404

Station #  111 TOKA   0.049  +/- 0.1510   40.323   36.478

Station #  112 EAK   -0.347  +/- 0.2033   40.736   43.607

Station #  113 DAD   -0.059  +/- 0.2897   39.908   32.753

Station #  114 EATA   0.051  +/- 0.2322   39.862   42.492

Station #  115 CNG    0.059  +/- 0.4144   38.215   39.299

Station #  116 MLTY   0.093  +/- 0.4489   38.326   38.425

Station #  117 CGC   -0.190  +/- 0.3360   37.747   37.296

Station #  118 AYK   -0.133  +/- 0.3356   37.415   36.807

Station #  119 AKO    0.110  +/- 0.1875   37.461   35.446

Station #  120 ZEY   -0.171  +/- 0.1818   38.239   26.503

Station #  121 BAGO   0.334  +/- 0.1305   37.991   30.789

Station #  122 ERE   -0.310  +/- 0.1761   40.045   28.891

Station #  123 AUKUT -0.068  +/- 0.7084   39.396   30.021

Station #  124 AUKIR -0.067  +/- 0.3822   39.287   30.531

Station #  125 AUMIH -0.100  +/- 0.3040   39.868   31.483

Station #  126 AUSIV  0.232  +/- 0.3810   39.440   31.539

Station #  127 CUSAR  0.001  +/- 0.1349   39.418   36.244

Station #  128 CUALT -0.072  +/- 0.1306   39.273   36.753

Station #  129 CUGUR -0.455  +/- 0.1309   38.724   37.272

Station #  130 CUKAN  0.008  +/- 0.1316   39.316   37.467

Station #  131 CUZAR -0.536  +/- 0.1568   39.888   37.771

Station #  132 KESN  -0.022  +/- 0.1474   40.826   26.680

Station #  133 AYVA  -0.078  +/- 0.1362   39.308   26.690

Station #  134 ECAT   0.532  +/- 0.2706   39.606   40.974

Station #  135 EPOS   0.251  +/- 0.2171   41.504   42.728

Station #  136 VMUR  -0.022  +/- 0.2385   38.989   43.572

Station #  137 BNGL  -0.134  +/- 0.2613   38.952   41.150

Station #  138 TNCL  -0.120  +/- 0.1878   39.115   39.542

Station #  139 ORDU  -0.039  +/- 0.1875   40.991   37.857

Station #  140 PAL   -0.107  +/- 0.7099   38.699   39.918

Station #  141 BTMN  -0.447  +/- 0.1545   37.891   41.270

Station #  142 SAMS  -0.234  +/- 0.1940   41.361   36.187

Station #  143 SURC  -0.351  +/- 0.1947   36.879   38.613

Station #  144 COAL   0.009  +/- 0.1315   40.255   34.986

Station #  145 AVNS  -0.074  +/- 0.1325   38.806   34.847

Station #  146 KERG   0.164  +/- 0.1434   37.415   34.125

Station #  147 BGOL   0.173  +/- 0.2158   38.988   40.671

Station #  148 REFA   0.125  +/- 0.1791   39.906   38.765

Station #  149 KKUL  -0.037  +/- 0.1405   39.171   32.917

Station #  150 KAND  -0.147  +/- 0.1453   41.094   30.192

Station #  151 SAHE  -0.072  +/- 0.1509   40.853   30.854

Station #  152 BTIN  -0.036  +/- 0.1731   41.636   32.250

Station #  153 KMER   0.219  +/- 0.1684   37.769   32.368

Station #  154 AKSY   0.143  +/- 0.1495   38.650   33.761

Station #  155 AUBOZ -0.008  +/- 0.4531   39.911   30.032

Station #  156 BUY   -0.151  +/- 0.2685   40.853   29.118

Station #  157 KEPZ   0.197  +/- 0.1737   36.902   31.604

Station #  158 YAHY   0.121  +/- 0.1663   38.095   35.363

Station #  159 CTAK   0.035  +/- 0.1904   40.648   34.791

Station #  160 BASK   0.127  +/- 0.2627   38.047   44.011

Station #  161 CMDR  -0.160  +/- 0.1602   40.494   32.475

Station #  162 ADCV   0.024  +/- 0.2634   38.808   42.725

Station #  163 YOVA   0.048  +/- 0.4546   37.587   44.290

Station #  164 SRTM  -0.214  +/- 0.2696   37.991   41.922

Station #  165 HANI  -0.077  +/- 0.1895   38.415   40.400

Station #  166 ISPR   0.282  +/- 0.5810   40.489   41.008

Station #  167 ELBS  -0.364  +/- 0.1679   38.324   37.133

Station #  168 SAIM  -0.043  +/- 0.1764   37.977   36.082

Station #  169 BAYC   0.045  +/- 0.1453   39.741   26.547

Station #  170 TAVA   0.206  +/- 0.1723   37.466   28.913

Station #  171 KZIL   0.212  +/- 0.1580   38.257   30.141

Station #  172 BRDR   0.072  +/- 0.1558   37.696   30.061

Station #  173 DOGA   0.201  +/- 0.1566   38.106   31.668

Station #  174 SINO  -0.150  +/- 0.2515   42.018   35.202

Station #  175 ODTU   0.098  +/- 0.1815   39.868   32.793

Station #  176 STEP  -0.274  +/- 0.1680   39.377   27.718

Station #  177 GEYV  -0.186  +/- 0.1765   40.484   30.296

Station #  178 HEKM  -0.223  +/- 0.1638   38.879   37.959

Station #  179 BAYB  -0.086  +/- 0.2223   40.258   40.257

Station #  180 IGDI   0.258  +/- 0.3590   39.868   44.078

Station #  181 GULN   0.095  +/- 0.2253   36.184   33.543

Station #  182 SANL  -0.231  +/- 0.2324   37.171   38.989

Station #  183 SMV1  -0.091  +/- 0.3177   39.340   29.024

Station #  184 SIMV   0.242  +/- 0.4486   39.109   29.024

Station #  185 GZT   -0.207  +/- 0.5031   37.355   37.561

Station #  186 DALY  -0.104  +/- 1.0111   36.816   28.653

Station #  187 FETY   0.383  +/- 1.0102   36.635   29.083

Station #  188 KBUK   0.098  +/- 0.5846   41.236   32.627

Station #  189 CLDR   0.361  +/- 1.0600   39.144   43.917

Station #  190 USAK   0.117  +/- 0.3635   38.714   29.018

Station #  191 AKUM   0.144  +/- 0.4170   36.323   30.343

Station #  192 AFYO  -0.098  +/- 0.3858   38.787   30.299